Setup RTDE:

- Set IP_robot=Gateway_computer vice verca

- Load URServerScript.script as Programm on teach pendant

- Enable Remote Control on Robot

- Turn on Robot

- Setup Environment on Matlab (Add-Ons Manage Add-Ons Setup Robotic System Toolbox Support Package)

Other Commands:

Send HighLevel Commands (popup hello world, brake release) to port 29999

executeUrScriptCommand(ur, movej(

Matlab:

- RTDE Website not correct, functions like sendJointConfigurationAndWait are not implemented in the Script provided by matlab, these functions dont work. They are identical tot he ones used by ROS2, when Opening them in Matlab they also refer to a ROS setup.

- No URCap for latest MATLAB release

- Check Ports on RTDE (30001 is slow 30004 is fast) default on sendURScript is 30001 for urRTDEClient ist 30004 (documentation)